AUGUST, 2024 EXAMINATION SERIES

COURSE: CWET

LEVEL: 1.2

SUBJECT: WASTEWATER

SECTION A

ANSWER ALL QUESTIONS IN THIS SECTION

| 1 | Night soil is…… - Soil containing urea - Human excreta, animal excreta and urea - Human excreta - Animal excreta | 1 mark |
| --- | --- | --- |
| 2 | Chlorination of wastewater is done for the removal of - Suspended solids - Pathogens - Sediments - Hardness | 1 mark |
| 3 | Solid content of wastewater is about - 10 % - 0.1 % - 1 % - 5% | 1 mark |
| 4 | The sewer which received discharge from two or more main sewers, is known as - A trunk sewer - An outfall sewer - A main sewer - An intercepting sewer | 1 mark |
| 5 | _________ is simply detaining water for a sufficient time. - Coagulation - Flocculation - Sedimentation - Filtration | 1 mark |
| 6 | What are the structures that are constructed at suitable intervals along the sewerage system called? - Manholes - Catch basins - Pumps - Sewer appurtenances | 1 mark |
| 7 | The sewer pipes ……… - Carry sewage as gravity conduits - Are designed for generating self-cleansing velocities at different discharge - Should resist the wear and tear caused due to abrasion - All the above | 1 mark |
| 8 | ## Pick up the correct statement from the following: - Deposition of sediment in the reservoir, reduces the capacity of the reservoir - Large solids carried along the river bed, is known as bed load - The fine sediment deposits are generally near the face of the dam - All the above | 1 mark |
| 9 | Identify the component above - Gate valve - Relief valve - None return valve - None of the above | 1 mark |
| 10 | Identify the component above - Gate valve - Relief valve - None return valve - None of the above | 1 mark |
| 11 | ## In a rapid sand filter, air binding is caused due to excessive - Turbidity - Negative pressure - Pressure - Water pressure | 1 mark |
| 12 | ## Which water distribution system is suitable for rectangular cities - Dead end system - Radial system - Ring system - Grid iron system | 1 mark |
| 13 | ## The detention period for plain sedimentation water tanks, is usually - 8 to 16 hours - 16 to 24 hours - 4 to 8 hours - 24 to 36 hours | 1 mark |
| 14 | ## Air valves are generally provided in pressure pipes of water supply - Near service pipes - At pipe junctions - At summits - At low points | 1 mark |
| 15 | Which of the following is used to measure the difference in pressure between two points in a pipe, or in two different pipes? - Single Column Manometer - Piezometer - Differential Manometer - Simple Manometer | 1 mark |
| 16 | The pressure head of liquid is - h = p/wg - h = p/w - h = p.w - h = w/p | 1 mark |
| 17 | The total energy line lies over the hydraulic gradient line by an amount equal to the…………………………… - Pressure head – Velocity head - Pressure head + Velocity head - Pressure head - Velocity head | 1 mark |
| 18 | The value of standard Atmospheric pressure is - 101.3 KPa - 10.13 KPa - 1.013 Kpa - 1013 KPa | 1 mark |
| 19 | Orifice and Mouthpieces are used to measure - the density of the liquid - the pressure of the liquid - the rate of flow of liquid - the velocity of liquid | 1 mark |
| 20 | In fluid mechanics, the continuity equation is a mathematical statement embodying the principle of - Conservation of mass - Conservation of energy - Conservation of weight - Conservation of momentum | 1 mark |

SECTION B 40 MARKS

ATTEMPT ALL QUESTIONS IN THIS SECTION

| 21 | Name the two main components of an activated sludge system. | 2 marks |
| --- | --- | --- |
| 22 | Differentiate between relief sewer and flight sewer. | 2 marks |
| 23 | Highlight three parts of a Shit Flow Diagram. | 3 marks |
| 24 | State the three necessities of sewer appurtenances. | 3 marks |
| 25 | Mention two reasons why most rural areas are not sewered. | 2 marks |
| 26 | Mention two types of sludge. | 2 marks |
| 27 | List 5 main processes of water treatment | 5 marks |
| 28 | Sketch the following symbols of water supply system - Closed water tank - Water meter - Fire hydrant | 3 marks |
| 29 | List any three methods of water treatment: | 3 marks |
| 30 | Describe a water intake structure | 2 marks |
| 31 | Differentiate between the following terms - Specific weight and specific density - Cohesion and adhesion | 4 marks |
| 32 | State three types of manometers | 3 marks |
| 33 | State Pascal’s law | 2 marks |
| 34 | State four types of hydraulic Co-efficient | 4 marks |

SECTION C

ANSWER ANY TWO QUESTIONS IN THIS SECTION

| 35 | - A 30mm diameter pipe branches, one branch being 20mm diameter and another 25mm diameter. If the velocity in the 20mm diameter pipe is 0.5m/s and that in the 25mm diameter pipe is - State four reasons why a faecal sludge is emptied and collected by vacuum trucks and not delivered to treatment. - Using a flow diagram, describe the necessary stages in a typical conventional wastewater treatment plant. - Mention any five problems encountered in sewage pumping. | 5 marks |
| --- | --- | --- |
|  |  | 4 marks |
|  |  | 6 marks |
|  |  | 5 marks |
| 36 | - You are the technical manager in Kitui water and sanitation company (KITWASCO). The company decides to supply water in a new region of Mutomo sub-county. The following census data is available for this area. Year; 1989; 1999; 2009; 2019. Population *1000; 50; 70; 85; 105. Determine the quantity of water that you need in water supply design to meet water demand for the population in Mutomo sub-county by the year 2059. (Hint: take per capita water consumption as 135l/d). - Using a well labeled sketch describe a canal intake. - A jet of water issues from an orifice of diameter 20mm under a head of 1m. what is the coefficient of discharge for the orifice, if actual discharge is 0.85 l/s . | 8 marks 7 marks 5 marks |
| 37 | - A drum of 2m3 volume contains 8.5kN of oil when full. Determine: - Specific weight - Specific gravity - A tank 4m x 5m contains 3m deep of oil of specific gravity 0.8.Find: - Intensity of pressure at the base of the tank - Total pressure on the base of the tank - With the aid of a neat sketch, describe a partially separate sewerage system and state any two advantages of this system. - Using a well labeled sketch describe a Radial water distribution system - A pitot tube was inserted in a pipe to measure the velocity of water in it. If the water rise in the tube is at a level of 200mm. Find the velocity of water | 4 marks 4 marks 4 marks 5 marks 3 marks |

.