AUGUST, 2024 EXAMINATION SERIES

COURSE: CWET

LEVEL: Y2S2

SUBJECT: OPERATION AND MAINTENANCE OF CONVENTIONAL WATER SUPPLY INFRASTRUCTURE

SECTION A: COMPULSORY (20MARKS)

ATTEMPT ALL QUESTIONS IN THIS SECTION

| 1 | Which is not a coagulant in water treatment? - Alum - Magnesium carbonate - Chlorinated coppers - Mercury | 1 mark |
| --- | --- | --- |
| 2 | Which one of the following is not a component of water supply? - Intake - Reservoir - Tipper - Consumer taps | 1 mark |
| 3 | What is not the objective of operation and maintenance in water supply? - To promote equipment breakdown. - To provide safe drinking water. - To provide water as per designed quantity. - To provide water with adequate pressure. | 1 mark |
| 4 | Preventive maintenance does not include? - Proper lubrication - Checking of pumps and motors for leaks. - Bad housekeeping. - Proper valve operation check. | 1 mark |
| 5 | Which is not the aim of water softening? - To reduce soap consumption of water. - To increase scaling of boilers. - To improve food taste. - Reduce maintenance of plumbing fixtures. | 1 mark |
| 6 | What does not entail water supply infrastructure? - Collection - Treatment - Transmission - Contamination of water. | 1 mark |
| 7 | Success of operation and maintenance programme should result in the following except? - Decline of frequency shutdowns - Decrease in frequency of emergency plans. - Increase in operational costs - Increase in water availability. | 1 mark |
| 8 | Which one of the following is not a community water supply scheme? - sand dams - springs - oceans - shallow wells | 1 mark |
| 9 | The following materials should be provided to caretakers of dug wells. Which is the odd one out? - Bucket - Rope - Sludge - cement | 1 mark |
| 10 | The following are monthly O & M activities for a hand pump. Which one is not? - De-silting of dug wells repair of chain - Lubricate the components with grease periodically - Clear the drains - Record the water level with a rope-scale | 1 mark |
| 11 | The two major types of filters are; - Gravity filter and pressure filter - Sand and gravel filter - Slow sand and rapid sand filter - Small sand filter and large sand filter | 1 mark |
| 12 | Which among the following defines disinfection? - Addition of a coagulant - Formation of flocks - Settling of flocks - Addition of chlorine so as to deactivate disease causing micro-organisms | 1 mark |
| 13 | Treatment involves filtration, which among the materials is used as a filter media - Stones - Mesh - Sand - Aluminum sulphate | 1 mark |
| 14 | What is operation - Activities which does not involve delivery of services - Activities which involves delivery of services - Keeping equipments - Servicing equipments | 1 mark |
| 15 | Which of the following defines coagulation? - Addition of chlorine - Addition of alum - Settling of flocks - Formation of flocks | 1 mark |
| 16 | Which among the following is not a component of intake structure? - Conduits - Strainer - Screens - Air Valve | 1 mark |
| 17 | Which of the following is not a cause of leakage in transmission mains? - Bad quality of materials used - Good workmanship - Corrosion - Vandalism | 1 mark |
| 18 | Aeration is used for the following purposes except: - Carbon dioxide reduction (de-carbonation) - Oxidation of iron and manganese of many well waters (oxidation tower) - Removal of turbidity - Effective method of bacteria control | 1 mark |
| 19 | Failure of pumps results from? - Incorrect design - Proper construction. - Proper operation - Adequate well development | 1 mark |
| 20 | Methods used for mixing of coagulant are numerous. Name an exception from the list. - Hydraulic mixing - Hand mixing - Mechanical mixing - Diffusers and grid system | 1 mark |

SECTION B: COMPULSORY (40 MARKS)

ATTEMPT ALL QUESTIONS

| 21 | Provision of spares and tools is an activity for good O&M in water supply. Name any three spare parts. | 3 marks |
| --- | --- | --- |
| 22 | State two operation and maintenance activities for water distribution system. | 2 marks |
| 23 | Explain each of the following terms: - Crisis maintenance (2mks) - Corrective maintenance with regard to water supply infrastructure (2mks) | 4 marks |
| 24 | Distinguish minor and major repairs with regard to hand pump maintenance. | 4 marks |
| 25 | A checklist is usually used to check tube well performance. Provide four items in the list. | 4 marks |
| 26 | State three operation and maintenance activities for a sand dam. | 3 marks |
| 27 | Name three requirements of a good disinfectant | 3 marks |
| 28 | List THREE methods of water treatment | 3 marks |
| 29 | Explain the coagulation and flocculation as used in water treatment | 6 marks |
| 30 | Provision of spares and tools is an activity for good O&M in water supply. Name any three spare parts. | 3 marks |
| 31 | State TWO methods of disinfecting water | 2 marks |
| 32 | Give three aspects to be considered in appraising water resources | 3 marks |

SECTION C (40 MARKS)

ANSWER ANY TWO QUESTIONS

| 33 | Pumping system is an important component in water supply infrastructure. With regard to this, provide: - Record maintenance for operation and maintenance of pumping machinery. (3mks) - Basics on its operation. (3mks) - Monthly preventive maintenance. (3mks) - Provide 4 maintenance activities for an unlined canal. (4mks) - State three daily O & M activities for a slow sand filter (3mks) - Valves forms part of transmission system. State two maintenance activities for: - Foot valve (2mks) - Butterfly valve (2mks) | 20 marks |
| --- | --- | --- |
| 34 | Using diagram, Describe the components of Reverse Osmosis Machine and explain the process of water purification | 20 marks |
| 35 | Water treatment removes undesirable constituents from water and disposes them in a safest manner. - Explain two resources required for operation and maintenance of a slow sand filter. (4mks) - The chlorine dosage for water is 1.9 mg/L. If the chlorine residual after 25 minutes contact time is found to be 0.6 mg/L, what is the chlorine demand, in mg/L? (4mks) - Discus four consequences of poor operation and maintenance of water supply facilities (8mks) - Explain two factors considered in operation and maintenance of water treatment facilities (4mks) | 20 marks |