AUGUST, 2024 EXAMINATION SERIES

SUBJECT: SET UP CUSTOMER SERVICE LINES

COURSE: CWET

LEVEL: Y2 S2

| SECTION A (20 MARKS) ANSWER ALL QUESTIONS |  |  |
| --- | --- | --- |
|  | Of the following which one is not a transmission line appurtenance - Chamber - Man hole - Soak pit - Wash out valve | (1mk) |
|  | The following are construction plant, which is suitable for trenching - Dozer - Grader - Crawler Excavator - Tipper truck | (1mk) |
|  | The following are pipe flow water parameters except - Hydrostatic pressure - Surface tension - Dynamic viscosity - Water flow Orifice | (1mk) |
|  | ### The colour code for potable and waste water is: - Red and blue - Green and yellow - Blue and green - Yellow and green | (1mk) |
|  | ### In general, why is no shoring required for water trenches? - They are planned for stable ground - They are usually less than 5ft deep - Sloping is required instead of shoring - Shielding is required instead of shoring | (1mk) |
|  | ### Choose type of trenching applied in loosed ground. ### Sloping ### Scaled ### Shoring ### shielding | (1mk) |
|  | ### If you lay pipe over base that is not fully compacted what exactly might happen. ### Nothing will occur ### Pipe may roll when laid ### Backfill will damage the pipe ### It will cause leaks at joints | (1mk) |
|  | ### Identify what is primarily used as warning for trench. - Flags - Flash lights - Signs - Tapes | (1mk) |
|  | ### If a leak is found during water testing of sewer pipe, what needs to be checked for leaking? - Manholes and its joints - Pipe and its joints - Plugs - A & B only | (1mk) |
|  | ### Which of the following must be considered while conducting a pipeline survey. ### Density ### Alignment of the curves ### Specific gravity ### Atmospheric condition | (1mk) |
|  | Among these survey tools which is most advanced - GIS and total station - Walking survey and chainage - Theodolite and staff survey - Levelling and traverse survey | (1mk) |
|  | Before installing new connection, which one is the best way to Obtain information about existing lines. - Reports and maps from water utilities - Daily Newspaper - Word of mouth - Observation and inspection | (1mk) |
|  | According to IS10500: 2012 for drinking water in the absence of alternate source of water,the permissible chloride and sulphate in mg/l respectively are: - 250 and 200 - 1000 and 400 - 200 and 250 - 500 and 1000 | (1mk) |
|  | Identify the device shown in the diagram - Pipe cutter - Drill bit - Pipe wrench - Pipe vice. | (1mk) |
|  | State a fitting that helps pipe to branch off at 90 degrees. - Elbow. - Plug. - Tee branch. - Coupling. | (1mk) |
|  | Identify fitting used to connect pipelines. - Reducer. - Hexagonal nut. - Pipe nipples. - Union. | (1mk) |
|  | The following are classes of pipes except... - Class E. - Class A. - Class B. - Class H. | (1mk) |
|  | Which one of these materials is used to manufacture non return valve - Mild steel - Aluminium - Bronze - Lead | (1mk) |
|  | Identify material that is not used to make pipes. - Lead - Copper - Plastic - Iron. | (1mk) |
|  | Identify instrument used to test accuracy of water meters - Ammeter - Barometer - Testing bench - Thermometer. | (1mk) |
| SECTION B (40 MARKS) ANSWER ALL QUESTIONS |  |  |
|  | Define customer connection. | (2mks) |
|  | Outline four types of pipe joints. | (4mks) |
|  | Briefly explain the following activities: - Mapping customer connections. - Pipe line route selection. | (3mks) (3mks) |
|  | List six types of pipes based on material | (3mks) |
|  | Explain how VJ joint is used in pipe laying | (3mks) |
|  | State five important safety precautions during pipe connection | (5mks) |
|  | State two methods of clearing the pipeline route | (4mks) |
|  | Explain two reasons why we backfill pipe trench | (4mks) |
|  | Outline methods of advanced pipeline mapping and route selection. | (4mks) |
|  | Discuss pipelaying procedure | (5mks) |
|  | SECTION C CHOOSE ANY TWO QUESTIONS 40 MARKS |  |
|  | - Describe the procedure of testing PVC pipeline | (10mks) |
|  | - Describe 10 types of pipe fittings, draw their symbols and outline the function of each. | (10mks) |
|  | - Outline pipe trench excavation procedure and state ten tools and equipment used in performing such work. | (15mks) |
|  | - Explain the necessity of installing meters in customer connections | (5mks) |
|  | - Describe ten types of pipe fittings, draw their symbols and state the function of each. | (10mks) |
|  | - State five advantages of PVC pipes compared to GI pipes. | (5mks) |
|  | - Explain the following metering operations. |  |
|  | - Meter placing. - Meter sizing. - Meter servicing. - Meter testing. - Meter protection. | (5mks) |