AUGUST, 2024 EXAMINATION SERIES

SUBJECT: OPERATION AND MAINTENANCE OF ONSITE SANITATION FACILITIES

COURSE: DWSET

LEVEL: YEAR THREE SEMESTER TWO

SECTION A (40 MARKS)

ANSWER ALL QUESTIONS IN THIS SECTION

| 1 | Safe disposal of excreta is crucial for a number of reasons. State two. | 2 marks |
| --- | --- | --- |
| 2 | Explain the principle of operation of a trench latrine. | 2 marks |
| 3 | Several factors are considered while choosing an onsite facility. List three. | 3 marks |
| 4 | Operation and maintenance of a VIP latrine are of great significance. Outline three of such activities. | 3 marks |
| 5 | Most onsite sanitation superstructure comprise of locally available materials. List any three materials used. | 3 marks |
| 6 | For operation of an Aqua privy, several materials are needed. Mention two materials used for cleaning the slab. | 3 marks |
| 7 | Detailed monitoring is an important aspect of O&M. Provide three reasons in support of this. | 3 marks |
| 8 | Mention two scenarios in which it necessitates inspection of a septic tank. | 2 marks |
| 9 | i. Define rehabilitation with respect to onsite sanitation (2mks) ii. Based on onsite sanitation facilities provide two examples of rehabilitation works. (4mks) | 6 marks |
| 10 | One tool used for storage and disposal of urine is jerrycans. Explain its appropriateness. | 3 marks |
| 11 | For a bio digester to operate well, it has to observe some general rules. Name four of such rules. | 4 marks |
| 12 | Prepare a maintenance schedule for a pit latrine. | 3 marks |
| 13 | A VIP latrine is an improvement of a pit latrine. State three advantages of the facility. | 3 marks |

SECTION B

ANSWER QUESTION 14 AND ANY OTHER TWO

| 14 | A septic tank is a majorly used onsite sanitation facility in recent times. With regard to this: - Define a septic tank. (2mks) - Briefly describe its working principle based on septic tank and drain field. (10mks) - Maintenance of the system ensures optimal performance. Outline three of such activities. (3mks) - You are assigned to perform an inspection of a septic tank. Outline various steps you will undertake to execute the task. (5mks) | 20 marks |
| --- | --- | --- |
| 15 | Maintenance of tools and equipment for onsite sanitation is of great importance for their operation and maintenance. Human-powered emptying and transport is one major method used. - Mention two ways used for emptying pits and tanks under this method. (2mks) - Discuss the appropriateness of this system. (4mks) - Discuss the working principle of a sludge hand pump. (5mks) - With respect to manual emptying of sludge, describe two ways in which the operators can minimize the effects of being in contact with sludge. (4mks) - Provide three operation and maintenance activities performed on this system. (3mks) - State two advantages of this system. (2mks) | 20 marks |
| 16 | Rehabilitation of onsite sanitation facilities is carried out often to ensure they serve the intended purpose. - As an operator assigned to carry out the rehabilitation works, briefly explain the steps you will undertake to achieve best results. (10mks) - O & M reports for onsite sanitation facilities has various activities in it. Name five of such activities. (5mks) - A pit latrine is the most used onsite sanitation facility in the villages. Based on this, describe: - The working principle and; (3mks) - State two operation and maintenance tasks performed to ensure it is working well. (2mks) | 20 marks |
| 17 | - You are tasked to perform an inspection and monitoring of a VIP latrine. Describe how you will carry out the exercise and prepare a maintenance schedule too. (10mks) - The conventional aqua privy is essentially a small septic tank located directly below a squatting plate, which has a drop pipe extending below the liquid level in the tank to form a simple water seal. Based on it principle of operation, describe: - Slab cleaning (2mks) - Sludge depth checking (2mks) - Emptying the septage (3mks) - Operation and maintenance activities. (3mks) | 20 marks |