AUGUST, 2024 EXAMINATION SERIES

COURSE: ICT

LEVEL: Y1S1

SUBJECT: APPLIED BASIC ELECTRONICS

SECTION A

- Draw  a well labeled electronic structure of an atom 			(3marks)

- Define the term inductor as used in electronics 				(1mark)

- John moves a magnet, upwards then downs, through a conductor connected to a galvanometer as shown in figure1. Explain what happens on the galvanometer on each occasion of the movements.  					(2marks)

Figure 1

- Copy the diagram and name the labeled components in the electrical circuit in Figure1							 (3marks)

Figure 2

- Calculate the combined capacitance of the network shown in the figure2

(3marks)

Figure 3

- Figure3 shows a battery with potential difference of 12V connected to two resistors with rating of 2ohms and 3ohms respectively. Determine the amount of current flowing through the circuit.					 (4marks)

Figure 4

- Contrast the conduction of electricity in insulators and semiconductors using the energy band theory  							(4marks)

- Distinguish between extrinsic and extrinsic semiconductors  		(2marks)

- Explain why the resistor shown in the figure5 is not suitable for measuring resistance in an AC Current. 						(4marks)

Figure 5

- Describe briefly how to make silicon a p type semiconductor 		(2marks)

- With the help of a diagram, describe a reverse biased PN junction. 	(4marks)

- An input voltage of 16V is applied to the resistor chain (see Figure 6) in the circuit .

Figure 6

Determine the reading on voltmeter when (1) switch S is open (11) when the S is closed.  									(4marks)

- Briefly describe any four types of electronic components 		(4marks)

SECTION B

QUESTION ONE

- Differentiate between the term  variable resistor and fixed resistor 											(2marks)

- Calculate the three digit codes for capacitors of value 		(4marks)

- 220F

- 82F

- Using color codes of the chart attached at the back page of this exam paper determine the value of the resistor resistance in figure 7.	 (4marks)

Figure 7

- The graph in figure 8 shows measurement of voltage (V) on the vertical axis, across a resistor of 10 ohms, in equal time(s) intervals on the horizontal axis.

Figure 8

Use this graph to Sketch a graph for the current flowing across the resistor verses time 								(6marks)

- Determine the value of the currents I, I1 and I2 flowing in the electrical circuit shown in Figure 9. 								(4marks)

Figure 9

QUESTION TWO

- Define the term semiconductor						(2mark)

- Explain the difference  between a conduction band and valance band in an atom         										(4marks)

- Illustrate the  atomic energy levels in a conductor and semi conductor affect conduction of electricity

(6marks)

- Draw a well labeled diagram to illustrate  a reverse biased  P-N junction												(4marks)

- Describe four applications of inductors 					(4marks)

QUESTION THREE

- If the p.d across a resistor is 9V and it carries a current of 3m A determine Its resistance 									(3marks)

- Copy each of the following symbol in your answer book and state what it  represented									(3marks)

- With the help of a diagram explain the charging process of a capacitor												(4marks)

- If the emf of the battery across each of the circuits is 9 volts and lamps, L1….L6, are of similar power rating.

Figure 10

Show, which lamps are:

- Dim

- Off

- Bright 										(6marks)

- Differentiate between variable and fixed capacitor giving an example of each. 											(4marks)

QUESTION FOUR

- Define the term binary digit 					(2mark)

- Explain how the computers uses binary numbers to save data

(4marks)

- Explain four advantages of binary Numbers in computing 	(4marks)

- Convert 0.815 into binary digit 					(4marks)

- Convert (11101)2 to BCD.						(6marks)