AUGUST, 2024 EXAMINATION SERIES

COURSE: PPFT

LEVEL:Y1 S2

SUBJECT: OPERATION AND MAINTENANCE

SECTION A

|  | QUESTIONS | MARKS | PC |
| --- | --- | --- | --- |
| 1 | ………. has a movable disc that rotates on a vertical shaft to stop the flow of water. - Sluice valve - Butterfly valve - Scour valve - Gate valve | 1 mark | 2.1 |
| 2 | What must you do before starting a centrifugal pump? - Check to see if the water seals are function properly - Examine the discharge valve - Follow correct lockout/tagout procedures - Properly prime the pump | 1 mark | 1.2 |
| 3 | What is the main function of a distribution storage facility? - To quickly route water to the treatment plant - To take care of daily demands and peak demands - To handle consumer complaints - To handle emergency supplies | 1 mark | 1.2 |
| 4 | Define the term corrective maintenance - The repair or adjustment of equipment of equipment and facilities that is done before deterioration takes place - Maintenance on a water storage facility following vandalism, localized corrosion, or splits due to extensive metal loss - Maintenance that is necessary when a problem already exists - Maintenance that tries to predict when a failure might occur so corrective action can be performed | 1 mark | 2.1 |
| 5 | …… is used to measure flows to domestic buildings. - water tank - drain pipe - water meter - rain gauge | 1 mark | 1.4 |
| 6 | In the design of water supply pipe networks, tree system type of distribution network is also known as: - radial system - ring system - grid iron system - dead end system | 1 mark | 1.2 |
| 7 | Where should thrust blocks be used? - When the ground soil is wet. - Where the pipeline changes direction. - Where flanged joints are used. - Both A and C. | 1 mark | 2.2 |
| 8 | Define the term preventive maintenance - Maintenance that tries to predict when a failure might occur so corrective action can be performed - The repair or adjustment of equipment and facilities that is done before deterioration takes place - The repair or adjustment of equipment and facilities that is necessary when a problem already exists - Maintenance on a water storage facility following vandalism, localized corrosion, or splits due to extensive metal loss | 1 mark | 2.1 |
| 9 | Which statement is correct with reference to grid iron water distribution system? - suitable for well-planned towns and cities - requires less number of sluice valves - cost of construction is low - all of the above | 1 mark | 1.2 |
| 10 | In which system of water supply, water is supplied only during fixed hours of the day? - Continuous supply - Low supply - Fixed supply - Intermittent supply | 1 mark | 1.3 |

SECTION B

|  | QUESTIONS | MARKS | PC |
| --- | --- | --- | --- |
|  | Identify six types of water storage facilities. | 6 marks | 1.2 |
|  | State five regular maintenance of water meters. | 5 marks | 1.4 |
|  | Describe the six steps of flushing the water supply pipeline. | 6 marks | 2.3 |
|  | What information about equipment should be included in records? | 6 marks | 2.2 |
|  | Describe the term “PIPE PUSH ON JOINT”. | 2 marks | 2.1 |
|  | List any eight pipe fittings. | 4 marks | 2.3 |
|  | List five maintenances for sluice valves. | 5 marks | 1.1 |
|  | Describe the sequence of repairing an underground pipe. | 5 marks | 1.2 |
| - . | Define the term “Pipe Appurtenances”. | 1 mark | 2.3 |